Project 3

Client: Federal Highway Administration (FHWA)

Report by: Savanna Flores, Gabriel Goeken, Connor Preble

Table of Contents:

Background	2

Objective	3

Methods	4

Results and Discussion	6

References	7

## Background

The United States depends on transportation systems as basic infrastructure elements, which enable economic growth and efficient movement of people and goods and create better living conditions for citizens. The systems need a reliable data-based understanding about how people travel and how traffic flows to achieve effective planning and operation. The Federal Highway Administration and other agencies spend significant resources on extensive data collection programs which provide essential support for their infrastructure planning and policymaking and system enhancement tasks.

The National Household Travel Survey and the Next Generation Simulation Data are the two primary datasets that transportation engineers use in their work. The National Household Travel Survey offers complete data about travel patterns used by American households, which includes their trip purposes and transportation methods, and their travel durations and distances. The data serves as a fundamental requirement for both macro-level transportation trend analysis and the development of long-term planning strategies (FHWA, 2017). The NGSIM database enables researchers to track vehicle movements in detail, which allows them to study specific driving behaviors, including driver speed and acceleration, and how drivers maintain distance from other vehicles (FHWA, 2006).

Engineers now consider the capacity to manage, visualize, and analyze transportation data from extensive dataset collection as an essential competency because of the current abundance of large data collections. Engineers use data visualization methods to find patterns, trends, and anomalies that remain hidden when examining unprocessed data. The Intelligent Driver Model and other simulation models create a system that allows researchers to study vehicle interactions during different traffic conditions (Treiber et al., 2000).

This project, assigned by the FHWA, uses Python-based tools to study and present transportation data through visual analysis. The objective of the project is to assist transportation planners by delivering visualizations that show important information with simulation results that improve their decision-making abilities.

## Objective

The primary objectives of this project are as follows:

- To analyze transportation datasets by extracting meaningful variables that represent travel behavior and traffic dynamics.

- To create clear and effective visualizations that communicate key trends and insights to both technical and non-technical audiences.

- To evaluate driving behavior through time-series analysis, providing a deeper understanding of vehicle dynamics.

- To implement and validate a simulation model (IDM) for reproducing real-world vehicle-following behavior using NGSIM data.

- To demonstrate the efficiency of Python-based data analysis, including documenting time spent on tasks to compare with traditional tools such as Excel.

- To support transportation planning decision-making by delivering actionable insights derived from data analysis and simulation.

## Methods

Many of the graphs presented with our code could potentially be created by hand or within Excel and the dataset. To make this easier, though, writing a proper Python code ensures that all variables are correctly accounted for, and it is easier in the sense of creating many graphs containing different variables. With respect to the bar chart created, per client request, the variables chosen were comparing the quantity of car types compared to their fuel consumption. These variables were picked to understand the general population’s fuel needs to properly provide enough fuel stations for each vehicle type. This bar chart was created from data from the NHTS dataset, isolating the x-axis containing each fuel type, and the y-axis containing the number of vehicles in the population that consumed that specific fuel type. The .value_counts() function was utilized to properly count each car type that is applied to each fuel type. This properly provided a good idea of the dispersion of fuel types in the population. Also, a color palette is applied to this plot to show the use of customization in the plots and graphs created.

The histogram that was provided, per the client's request, showed the proper distribution of vehicles with respect to their age. Understanding the distribution age of vehicles in the population would be critical to know when considering maintenance of these vehicles. To properly create the histogram, the plt.hist() function was used with respect to the vehicle age column within the NHTS dataset. Within the code, the number of bins was chosen to be 30 to give a good, representative distribution of the data. The x-axis was chosen to correspond to the age of the vehicle in years, and the y-axis was the “count,” or number of vehicles within the age range, with respect to the bins.

The boxplot created using Python code shows the vehicle age with respect to its specific make. This could be useful in understanding the types of cars that are popular now and could be used to make predictions of what cars could be popular within the next few years. This boxplot was created by using the .boxplot() function with respect to the NHTS dataset. The columns of the boxplot were set to be vehicle age with respect to their make within the code. The plt.xticks function was used to rotate the column labels by 90 degrees to better read them. The x label was defined as the vehicle’s make, and the y-axis was defined as the vehicle’s age in years, which properly plotted the boxplot for each vehicle make by age. The .tight_layout() function was also utilized to make the boxplot fully appear when shown, as some labels could be cut off without it.

The time-series plots were created so that the first plot showed follower acceleration vs time and the second one displayed follower speed vs time. Both provide an understanding of traffic patterns and can be utilized to better adapt to traffic and road design in the future. Both plots were created very similarly, only changing the variable shown on the y-axis, being accelerated in the first graph, and speeding in the second. They both referenced the time on the x-axis. The sns.lineplot() function was used to reference time on the x-axis, and acceleration/speed on the y-axis. This was also referencing the data within the NGSIM dataset.

For the IDM simulation, an IDM acceleration was first created by defining variables with a specific acceleration function. To complete this, an equation to find the minimum gap was created and came out to “s_star = s0 + vT + (vdelta_v) / (2np.sqrt(ab)).” These variables were defined early in the function that defined the IDM acceleration and are defined beforehand, later, when the simulation is run. To create the actual acceleration function, this function is used: acceleration = a*(1 - ((v/v0)**delta) - ((s_star/s)**2)). Again, these values will be defined beforehand as parameters or pulled from the dataset during the true simulation. Now, regarding the NGSIM dataset, new variables are added to simulate a vehicle following a leader vehicle. Trajectory numbers, leader position, leader speed, and time are all to be extracted from the dataset at a given point. Specifically, for the time function, the change in time is to be considered, so the function “dt = time[1] - time[0]” is utilized.

## Results and Discussion

As a result of creating a bar chart comparing the count of vehicles by fuel type, we discovered that gasoline has the highest number of vehicles that use this fuel type, according to the data collected from NHTS. Understanding what fuel type is most in demand is crucial for the transportation industry because it affects economic growth and environmental considerations. Knowing these factors is important because the choice of fuel can impact the cost of transportation and overall living costs. While understanding the environmental impact fuel usage can have, knowing helps in assessing the environmental footprint of transportation.

Furthermore, the histogram variables selected show the distribution of vehicles by their age. Having a high average vehicle age is not always bad, but the maintenance and repairs can become costly as a vehicle ages. Knowing the average vehicle age is important for the transportation industry because being able to monitor the performance of cars helps to make improvements, identify strengths, and weaknesses.

In addition, the boxplot graph illustrates vehicle age by making. The relevance this brings to the transportation industry can also help monitor improvement and keep businesses in competition with each other. A higher vehicle age can also show the sturdiness of a vehicle and make customers more inclined to invest in one vehicle over another.

The first time-series plot shows and analyzes follower acceleration vs time, while the second time-series plot is follower speed vs time. The importance of this time series helps the transportation industry make safer decisions about braking distance and travel speeds, which are critical for preventing accidents. It also allows engineers to design vehicles and infrastructure that can safely accommodate the expected acceleration of vehicles. Understanding these time series’ importance can help with resource optimization and being able to know the quality of service on multi-lane roads.

## References

- Federal Highway Administration (FHWA). (2006). Next Generation Simulation (NGSIM) Vehicle Trajectories and Supporting Data. U.S. Department of Transportation.

- Federal Highway Administration (FHWA). (2017). National Household Travel Survey (NHTS) User Guide. U.S. Department of Transportation.

- Treiber, M., Hennecke, A., & Helbing, D. (2000). Congested traffic states in empirical observations and microscopic simulations. Physical Review E, 62(2), 1805–1824.

- KPI Depot. (2025). Average Vehicle Age KPI. KPI Depot LLC.